This MSDS is being provided to your company for the purpose of providing current health and safety information to your management and for your employees who work with this product.  Please read the information on these sheets and then provide this information to those people at your company whose responsibility it is to comply with FEDERAL, STATE and COMMUNITY

If Hi-Gear Products, Inc. considers the formula of this product to be a trade secret, the exact chemical names of the ingredient(s) and the percentages in which they are combined will not appear in the body of this sheet.  The exact composition is available upon request to physicians, industrial hygienists and other health professionals.

IN CASE OF AN EMERGENCY CONTACT CHEM-TREK AT: RIGHT TO KNOW regulations.  Also, make this information available to any employee who requests it.	                        	       USA  800-424-9300

INTERNATIONAL  703-527-3887

## SECTION – 1 CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

Trade Name:	HG3319, SP3319 Starting Fluid

Chemical Family:	50% Ether

## SECTION 2- COMPOSITION/INFORMATION ON INGREDIENTS

| Ingredient/Cas No. | Wt.% | OSHA PEL TWA | OSHA PEL Ceiling Limits | ACGIH TLV TWA | ACGIH TLW STEL |
| --- | --- | --- | --- | --- | --- |
| Ethyl Ether 60-29-7 | 30-40 | 400 PPM | None Established | 400 PPM | 500 PPM |
| Heptane 142-82-5 | 20-30 | 500 PPM: 2000 mg/m3 | None Established | 400 PPM: 1640 mg/m3 | 500 PPM |
| Propane 74-98-6 | 5-15 | 1000 PPM | Not known | 2500 PPM | Not Known |
| Carbon Dioxide 124-38-9 | 2-10 | 5000 PPM (exposures <10,000 PPM to be cited de minimus | Not Known | 5000 PPM | 30,000 PPM |
| ISO-Butane 75-28-5 | 2-10 | None Established | None Established | None Established | None Established |
| Lubricating Oil 64742-52-5 | 0.5-5 | Not known | Mist 5mg/m3 | TLV Mist 5 mg/m3 8 HR | Not Known |

This product contains trace amounts of (<15 PPM) of Butylated hydroxytoluene (BHT) as an inhibitor to prevent or reduce the information of potentially explosive peroxides.

## SECTION 3- HAZARDS IDENTIFICATION

Emergency Overview:	Danger: Extremely flammable.  Breathing high concentrations of vapor or mist may cause nausea, vomiting, central nervous system (CNS) depression and asphyxiation.  Symptoms may include headache, dizziness, blurred vision, slurred speech, memory loss, confusion, fatigue, loss of consciousness, convulsions, paralysis, or coma.  This material is irritating to skin eyes, and respiratory tract.  Keep away from heat, sparks and flame.  Prolonged or repeated inhalation or ingestion may result in kidney and liver changes.

## SECTION 4- FIRST AID MEASURES

Eye contact:	In case of contact, or suspected contact, immediately after flushing.  Do not permit victim to rub eyes.

Ingestion:	If swallowed, call a physician immediately.  Only induce vomiting at the instruction of a physician. Never give anything by mouth to an unconscious person.  Get medical attention!  If vomiting occurs, keep head lower then hips to prevent aspiration.

Inhalation:	If inhaled, remove to fresh air.  If not breathing give artificial respiration, preferably mouth to mouth. If breathing is difficult give oxygen. Get medical attention.

Skin contact:	Wash with soap and water for 15 minuets. If irritation persists or signs of toxicity occur, seek medical attention.  Remove contaminated clothing and launder before reuse.

## SECTION 5 – FIRE AND EXPLOSION INFORMATION

Flammable Properties

Trade Name:		Hi-Gear 50% Starting Fluid (HG3319)

Flash Point 0 F (0 C)		- 76 F

Flash Point Method 		TAG Closed Cup

Flammable Limits in Air- Lower (%):	1.2% (Lowest Component)

Flammable Limits in Air-Upper (%):	6.7% ( Lowest Component)

Auto ignition Temperature oF (oC):	356 Fo ( Lowest Component)

Extinguishing Media:		Dry chemical.  Carbon  dioxide.  Alcohol foam.  Use water spray to keep containers cool that are exposed to heat of flames.

Protection of firefighters:

Special Firefighting Procedures:	Wear approved positive-pressure self-contaminated breathing apparatus and protective clothing.  Vapor may cause flash fire.  Fight from  a maximum distance or use unmanned hose holders or monitor nozzles.  Containers can build up pressure if exposed to heat; cool with flooding quantities of water until well after the fire is out.  Withdraw immediately in case of rising sound from venting safety devices or discoloration of vessel.

Hazardous Combustion Products:	Carbon Dioxide.  Carbon Monoxide.

Aerosol comments:		NFPA Level 3 Aerosol

## SECTION 6 – ACCIDENTAL RELEASE MEASURES

Personal Precautions:	Wear approved protective clothing and equipment to prevent skin and eye contact.  Contain any liquid from leaking containers.  Avoid all sources of ignition; heat, sparks and open flames.  Do not puncture, incinerate containers.  Contents under pressure  wear proper protective equipment as specified in the protective equipment section.  Remove sources of ignition.  Leaking containers should be removed to an isolated, well- ventilated area and transferred to other suitable containers.  Wipe, scrape, or soak up in an inert material and put in a container intended for flammable materials for disposal.

Environmental Precautions:	Do not allow to enter sanitary drains, sewers or surface waters.  Keep out of lakes, ponds or streams.

## SECTION 7- HANDLING AND STORAGE

Handling and storage:	Caution: Contents under pressure.  Keep away from open flame.  Use only in a well ventilated area.  Avoid breathing vapors, if exposed to high vapor concentration, leave area at once.  Avoid contact with skin and eyes.  Do not puncture, incinerate or store above 1200 F.  Exposure to high temperatures may cause bursting.  Do not store in the passenger compartment of an automobile.  Store in a cool, dry place, out of direct sunlight.

## SECTION 8- EXPOSURE CONTROLS/PERSONAL PROTECTION

Engineering Controls:	Use in a well ventilated area.  Local exhaust ventilation as necessary to maintain exposures to within applicable limits.

Eyes:		Chemical goggles; also wear face shield if splashing hazard exists.

Skin protection:	Avoid skin contact.  Wear protective clothing and gloves.

Respiratory Protection:	Do not breathe mist or vapor.  Use in a well ventilated area,  appropriate respiratory protection shall be worn when applied engineering controls are not adequate to protect against inhalation exposure.

## SECTION 9- PHYSICAL AND CHEMICAL PROPERTIES

Appearance:	colorless to pale yellow liquid

Odor:		PUNGENT SWEET

pH Value:	Not Determined

Vapor Pressure:	Not determined

Vapor Density(Air=1):	>1.5 Approximate

Boiling Point (0F)	-44oF (Lowest Component)

Melting/Freezing Point:	Freezing-176oF (Ether)

Solubility in water:	PARTLY SOLUBLE

BULK Density at 200C:	Not Determined

Molecular Weight:	Mixture

Evaporation Rate:	Not determined

Viscosity:	Not Determined

Specific Gravity:	Not determined

VOC Content(%):	Not determined

Decomposition Temp:	Not determined

## SECTION 10 – STABILITY AND REACTIVITY

Chemical Stability:	Stable under normal conditions of handling, use and transportation.

Conditions to avoid:	Keep away from heat, sparks and flame.  Avoid any source of ignition.  Do not expose to heat or store at temperatures above 120oF.

Materials to avoid:	Contact with oxidizing agents.  Concentrated oxygen.  Nitric acid.  Avoid contact with chorine in the presence of light.

Hazardous Polymerization:	WILL NOT OCCUR

## SECTION 11- TOXICOLOGICAL  INFORMATION

Toxicological Data:

| Ingredients/CAS No. | Wt. % | Route | Species | Dose |
| --- | --- | --- | --- | --- |
| Ethyl Ether 60-29-7 | 30-40 | Inhalation | Mice | LC50 31000 ppm/30M |
| Heptane 142-82-5 | 20-30 | Inhalation | Rats | LC50 103 gm/m3/4H |
| Propane 74-98-6 | 5-15 | NA | NA | Not known. |
| Carton Dioxide 124-38-9 | 2-10 | NA | NA | Not known. |
| Iso-Butane 75-28-5 | 2-10 | Inhalation | Rats | LC50 57 pph/15M |
| Lubricating Oil 64742-52-5 | .5-5 | NA | NA | Not known. |

Carcinogenicity:

| Ingredients/CAS No. | Wt. % | IARC | NTP | OSHA |
| --- | --- | --- | --- | --- |
| Ethyl Ether 60-29-7 | 30-40 | Not Listed | Not Listed | Not Listed |
| Heptane 142-82-5 | 20-30 | Not Listed | Not Listed | Not Listed |
| Propane 74-98-6 | 5-15 | Not Listed | Not Listed | Not Listed |
| Carton Dioxide 124-38-9 | 2-10 | Not Listed | Not Listed | Not Listed |
| Iso-Butane 75-28-5 | 2-10 | Not Listed | Not Listed | Not Listed |
| Lubricating Oil 64742-52-5 | .5-5 | Not Listed | Not Listed | Not Listed |

## SECTION 12 – ECOLOGICAL INFORMATION

Ecological testing has not been conducted on this product.

## SECTION 13 – DISPOSAL CONSIDERATION

Waste Classification:	Residues and spilled material are hazardous waste due to ignitability.

Waste Management:	Not determined.

Disposal should be made in accordance with federal, state and local regulations.

## SECTION 14 – TRANSPORTATION INFORMATION

U.S. DOT:

Proper Shipping Name:	Consumer Commodity

Hazard Class:	ORMD

UN/NA Number:	Not Applicable

DOT Packing Group:	Not Applicable

IMDG:

Proper Shipping Name:	Aerosols (limited quantity)

Hazard Class:	2.1

Hazard Subclass:	Not determined.

UN No.:	UN 1950

Packing Group:	Not Determined

Marine Pollutant:	No

## SECTION 15 – REGULATORY  INFORMATION

US Federal Regulations:

| Ingredients/CAS No. | Wt. % | SARA 313 | SARA 302 | RQ | TPQ |
| --- | --- | --- | --- | --- | --- |
| Ethyl Ether 60-29-7 | 30-40 | Not Listed | Not Listed | 100 lbs. | NA |
| Heptane 142-82-5 | 20-30 | Not Listed | Not Listed | NA | NA |
| Propane 74-98-6 | 5-15 | Not Listed | Not Listed | NA | NA |
| Carbon Dioxide 124-38-9 | 2-10 | Not Listed | Not Listed | NA | NA |
| Iso-Butane 75-28-5 | 2-10 | Not Listed | Not Listed | NA | NA |
| Lubricating Oil 64742-52-5 | .5-5 | Not Listed | Not Listed | NA | NA |

Hazardous per OSHA 29 CFR 1910.1200

SARA 311/312 Hazard Categories:  Immediate/Acute, Delayed/Chronic, Fire

State Regulations:

| Ingredients/CAS No. | Wt. % | California Prop. 65 Cancer List | California Prop. 65 Developmental Toxicity | California Prop. 65 Reproductive Female | California Prop. 65 Reproductive Male |
| --- | --- | --- | --- | --- | --- |
| Ethyl Ether 60-29-7 | 30-40 | Not Listed | Not Listed | Not Listed | Not Listed |
| Heptane 142-82-5 | 20-30 | Not Listed | Not Listed | Not Listed | Not Listed |
| Propane 74-98-6 | 5-15 | Not Listed | Not Listed | Not Listed | Not Listed |
| Carbon Dioxide 124-38-9 | 2-10 | Not Listed | Not Listed | Not Listed | Not Listed |
| Iso-Butane 75-28-5 | 2-10 | Not Listed | Not Listed | Not Listed | Not Listed |
| Lubricating Oil 64742-52-5 | .5-5 | Not Listed | Not Listed | Not Listed | Not Listed |

U.S. TSCA:	The components of this product are listed on the TSCA Inventory.

## SECTION 16 – OTHER INFORMATION

General Notes:  Do not allow undiluted material or large quantities to reach groundwater, bodies of water or sewer system.

Date entered:  09-24-99

Date revised:  8/4/06; 08/16/2006

Hi-Gear Products, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy.  Individuals receiving this information must exercise their independent judgment in determining its appropriateness for a particular purpose. Hi-Gear Products, Inc.   makes no representations or warranties, either expressed or implied, of merchantability, fitness for a particular purpose with respect to the information set forth herein or to the product to which the information refers.  Accordingly, Hi-Gear Products, Inc. will not be responsible for damages resulting from use of or reliance upon this information.